# Lab No. 6 & 7

# Conditional Processing

# A: Boolean & Comparison Instructions

# B: Conditional Jumps & Loops

# Lab Work

- Write the following code and explore the functioning of loop by debugging it:

.DATA

source  BYTE  "This is the source string",0

target  BYTE  SIZEOF source DUP(0)

.CODE

main PROC

mov  esi,0				; index register

mov  ecx, SIZEOF source		; loop counter

L1:

mov  al,	source[esi]		; get char from source

mov  target[esi],	al		; store it in the target

inc  esi				; increment index

loop L1				; loop for entire string

invoke	ExitProcess, 0	; to terminate the program

main ENDP

END main

- Write a program to calculate the sum of an array of 16-bit integers having values;

short wordArray[6] = {100,200,300,400,500,600};

- Use AND instruction to convert lower-case characters to upper-case characters. Hint:

'a' = 0 1 1 0 0 0 0 1		'b' = 0 1 1 0 0 0 1 0		lower-case characters

'A' = 0 1 0 0 0 0 0 1		'B' = 0 1 0 0 0 0 1 0		upper-case characters

Code:

.386p

.MODEL flat, stdcall

.code

main proc

mov eax,01100001b

mov eax,01000001b

and eax ,eax

main endp

end main

Output:

- Use OR instruction to convert a binary digit to ASCII digit. Hint:

0   = 0 0 0 0 0 0 0 0		1   = 0 0 0 0 0 0 0 1		binary digits

‘0’ = 0 0 1 1 0 0 0 0		'1' = 0 0 1 1 0 0 0 1		ascii digits

Code:

.686p

.MODEL flat, stdcall

.code

main PROC

mov al, 00000001b

OR al, 00110001b

main endp

end main

Output:

- Compute the Max of unsigned EAX and EBX

- Compute the Min of signed EAX and EBX

# Exercise

- Write a program to find the first negative value in an array.

- Implement following block-structured IF statements in assembly;

if( var1 == var2 )

X = 1;

else

X = 2;

Code:

.386p

.model flat, stdcall

ExitProcess proto, dwexitcode: dword

.data

x byte ?

.code

main proc

mov eax,5

mov ebx,15

cmp eax,ebx

jne not_equal

equal:

mov x,1

not_equal:

mov x,2

main endp

end main

Output:

  - Treat the following variables as unsigned;

if( ebx <= ecx )

{

eax = 5;

edx = 6;

}

  - Treat the following variables as signed;

if (var1 <= var2) {

var3 = 10;

}

else {

var3 = 6;

var4 = 7;

}

  - Compound expression with logical AND;

if ((al > bl) && (bl > cl)) {X = 1;}

  - While loop

while( eax < ebx) { eax = eax + 1; }

Code:

.386p

.model flat, stdcall

ExitProcess proto, dwexitcode: dword

.data

.code

main proc

mov eax,5

mov ebx,10

L1:

cmp eax,ebx

jge finish

add eax,1

jmp L1

finish:

push 0

call ExitProcess

main endp

end main

Output:

Checked By: 								Date: